# Software Detail Design (SDD)

Standard: IEC 62304:2006 + A1:2015 
Device Name: AI-Based Lung Opacity Detection System 
Software Item / System: Chest X-Ray Diagnostic Pipeline 
Version: 1.0 
Document Owner: Bhavya Reddy 
Course: INFO7410 — Advanced Medical Device Software Engineering 
Institution: Northeastern University 
Instructor: Professor Bemin

## 1. Purpose

This Software Detail Design document describes the internal design of each software module in the AI-Based Lung Opacity Detection System in accordance with IEC 62304:2006 + A1:2015 Clause 5.4. It defines inputs, outputs, algorithms, decision logic, data structures, and error handling for all modules in the 6-level diagnostic pipeline. This document is traceable to the Software Development Plan v1.0 and Functional Requirements v1.2.

## 2. References

- IEC 62304:2006 + A1:2015 — Medical Device Software: Software Life Cycle Processes

- Software Development Plan v1.0

- Functional Requirements v1.2

- Project Proposal — AI-Powered Lung Opacity Detection System Using YOLOv8

- README — AI-Based Lung Opacity Detection System

- scripts/main.py — FastAPI Backend Implementation

- frontend/src/App.js — React Frontend Implementation

## 3. Software Safety Classification

| Module | IEC 62304 Class | Rationale |
| --- | --- | --- |
| CLAHE Preprocessing | Class A | Pure image enhancement — no clinical output |
| Lung Segmentation (OpenCV) | Class A | Mask extraction — no independent clinical decision |
| Level 1 — Binary Classifier | Class B | Misclassification could cause missed diagnosis |
| Level 2 — Disease Classifier | Class B | Incorrect class could influence triage |
| Level 3 — Opacity Detector | Class B | Detection failure affects downstream severity grading |
| Level 4 — Area Calculation | Class B | Inaccurate percentage directly affects severity |
| Level 5 — Severity Assessment | Class B | Incorrect severity affects clinical recommendation |
| Level 6 — Clinical Recommendation | Class B | Incorrect recommendation could delay or escalate triage |
| FastAPI Backend | Class B | Pipeline failure prevents delivery of diagnostic report |
| React Frontend | Class B | Display failure prevents radiologist from reviewing results |

Class B Justification: A licensed radiologist always confirms all outputs before clinical action. No module autonomously initiates clinical decisions, ruling out Class C.

## 4. System Architecture and Data Flow

The React frontend sends a PNG or JPG chest X-ray to the FastAPI backend via HTTP POST. The backend applies CLAHE preprocessing, runs AI inference, executes rule-based logic, and returns a structured JSON response with an annotated X-ray image.

User → React Frontend

→ HTTP POST /api/analyze (multipart/form-data)

→ FastAPI Backend

→ CLAHE Preprocessing + Lung Segmentation

→ Level 1 + Level 3 (run together — Level 3 is independent safety check)

→ Confidence Validation (cross-check L1 + L3)

→ [If NORMAL and confident → stop, return NORMAL result]

→ Level 2 → Level 4 → Level 5 → Level 6

→ JSON Response → React Frontend → Radiologist Review

Pipeline Bypass Conditions:

| Condition | Action |
| --- | --- |
| Level 1 = NORMAL, confidence > 85%, no boxes detected | Pipeline stops — return NORMAL result |
| Level 1 confidence < 80% AND boxes detected | Escalate to ABNORMAL |
| 2 or more boxes detected by Level 3 | Escalate to ABNORMAL regardless of Level 1 |
| No boxes detected by Level 3 | box_list is empty — Level 4 returns 0% |

## 5. Data Structures

### 5.1 Image Representations

| Representation | Type | Shape | Convention |
| --- | --- | --- | --- |
| PIL Image | PIL.Image | image.size = (width, height) | Width-first |
| NumPy Array | np.ndarray | (height, width, 3) | Height-first |
| Lung Mask | np.ndarray bool | (height, width) | Height-first |
| Opacity Mask | np.ndarray bool | (height, width) | Height-first |

PIL and NumPy use opposite dimension ordering. All functions accept img_w and img_h as separate parameters and apply them explicitly.

### 5.2 Bounding Box Format

Level 3 outputs normalized YOLO-format bounding boxes:

[center_x, center_y, width, height] — all values in range [0.0, 1.0]

### 5.3 JSON Response Structure

{

"level1": { "result": "ABNORMAL", "confidence": 94.2 },

"level2": { "disease": "lung_opacity", "confidence": 87.3,

"probabilities": { "Normal": 2.1, "COVID-19": 5.4,

"Lung Opacity": 87.3, "Viral Pneumonia": 5.2 } },

"level3": { "boxes_detected": 2, "location": "Both lungs", "is_bilateral": true },

"level4": { "affected_percentage": 38.5 },

"level5": { "severity": "MODERATE" },

"level6": { "priority": "HIGH (ORANGE)", "action": "Flag for urgent radiologist review",

"timeline": "Within 2-4 hours", "triage": "Urgent",

"notes": ["Monitor oxygen saturation"] },

"annotated_image": "<base64-encoded PNG string>"

}

If Level 1 returns NORMAL with high confidence, level2 through level6 are returned as null.

## 6. Module Detail Design

### 6.1 CLAHE Preprocessing

Safety Class: A | File: scripts/main.py | Traces to: FR-01

Input: Raw image as RGB numpy array (H × W × 3)

Algorithm:

- Convert RGB array to grayscale

- Apply CLAHE — clipLimit=2.0, tileGridSize=(8, 8)

- Convert enhanced grayscale back to RGB

Output: CLAHE-enhanced RGB numpy array — same shape as input

Design Decision: Operates entirely in memory — no disk I/O.

### 6.2 Lung Segmentation (OpenCV)

Safety Class: A | File: scripts/main.py | Traces to: FR-07

Input: CLAHE-enhanced RGB numpy array (H × W × 3)

Algorithm:

- Convert RGB to grayscale

- Apply Gaussian blur (5×5 kernel)

- Iterate threshold values: 60, 80, 100, 120, 140

  - Apply inverse binary threshold

  - Apply morphological CLOSE then OPEN (15×15 kernel)

  - Extract up to two largest contours with area > 5,000 pixels

  - Select threshold producing the largest valid mask (capped at 75% of image area)

- Return best mask as boolean numpy array (H, W)

Fallback: If best mask area < 100 pixels, return center 80% of image.

Output: Boolean numpy array (H, W) — True pixels represent lung tissue

6.3 Level 1 — Binary Classifier

Safety Class: B | Model: YOLOv8s-cls (5M parameters) | File: scripts/main.py | Traces to: FR-02

Input: CLAHE-enhanced RGB numpy array

Algorithm:

- Run YOLOv8s-cls inference

- Extract top-1 class: normal or abnormal

- Extract top-1 confidence score

Output: level1_class (string), level1_confidence (float)

Performance: 96.1% accuracy (target ≥90%) — Passed

Confidence Validation Cross-Check with Level 3:

| Level 1 Result | Level 3 Boxes | Final Decision |
| --- | --- | --- |
| NORMAL, confidence > 85% | 0 boxes | NORMAL — pipeline stops |
| NORMAL, confidence < 80% | ≥ 1 box | Escalate to ABNORMAL |
| Any | ≥ 2 boxes | Escalate to ABNORMAL |
| ABNORMAL | Any | ABNORMAL — pipeline continues |

### 6.4 Level 2 — Disease Classifier

Safety Class: B | Model: YOLOv8l-cls (36.2M parameters) | File: scripts/main.py | Traces to: FR-03, FR-04

Input: CLAHE-enhanced RGB numpy array (only if pipeline continues past Level 1)

Algorithm:

- Run YOLOv8l-cls inference

- Extract top-1 class and confidence

- Extract full probability distribution across all 4 classes

Classes: Normal, COVID-19, Lung Opacity, Viral Pneumonia

Output: level2_class (string), level2_confidence (float), probs (dict — all 4 class probabilities summing to 100%)

Performance: 98.0% overall accuracy (target ≥90%) — Passed

### 6.5 Level 3 — Opacity Detector

Safety Class: B | Model: YOLOv8m-det (25.8M parameters) | File: scripts/main.py | Traces to: FR-05, FR-06

Input: CLAHE-enhanced RGB numpy array — runs on every image regardless of Level 1 result

Algorithm:

- Run YOLOv8m-det inference with confidence threshold 0.25

- For each box with confidence ≥ 0.25:

  - Extract normalized coordinates [cx, cy, bw, bh]

  - Draw bounding box on original PIL image

  - Classify location: cx < 0.5 → left lung; cx > 0.5 → right lung

- Determine bilateral: left_lung AND right_lung = True

Output: box_list, location, is_bilateral, annotated_b64

Performance: 53.1% mAP50 (target ≥70%) — Below target. See Section 7.

### 6.6 Level 4 — Affected Area Calculation

Safety Class: B | File: scripts/main.py | Traces to: FR-07

Input: box_list, img_w, img_h, lung_mask (H × W bool array)

Algorithm:

- Build opacity mask (H × W) from bounding boxes

- Resize lung_mask if shape does not match (img_h, img_w)

- If lung area < 100 pixels → return 0.0

- Compute intersection and return percentage

Formula:

affected_percentage = (opacity_mask ∩ lung_mask).sum() / lung_mask.sum() × 100

Output: Float in range [0.0, 100.0]

### 6.7 Level 5 — Severity Assessment

Safety Class: B | File: scripts/main.py | Traces to: FR-08

Input: affected_percentage (float), is_bilateral (bool)

Decision Rules:

| Affected Area | Base Severity | Bilateral Adjustment |
| --- | --- | --- |
| < 25% | MILD | Upgrade to MODERATE |
| 25% – 50% | MODERATE | No change |
| > 50% | SEVERE | No change |

Output: severity — "MILD", "MODERATE", or "SEVERE"

### 6.8 Level 6 — Clinical Recommendation

Safety Class: B | File: scripts/main.py | Traces to: FR-09

Input: severity, affected_percentage, disease

Decision Rules:

| Condition | Priority | Action | Timeline | Triage |
| --- | --- | --- | --- | --- |
| SEVERE or affected > 60% | IMMEDIATE (RED) | Immediate radiologist review required | Within 30 minutes | Emergency |
| MODERATE or affected ≥ 30% | HIGH (ORANGE) | Flag for urgent radiologist review | Within 2–4 hours | Urgent |
| MILD or affected < 30% | ROUTINE (YELLOW) | Routine radiologist review | Within 24 hours | Standard |

Disease-Specific Notes:

- COVID-19 → "Recommend isolation precautions"

- MODERATE AND affected ≥ 30% → "Monitor oxygen saturation"

- SEVERE → "Consider ICU admission"

Output: Dict — priority, action, timeline, triage, notes

### 6.9 FastAPI Backend

Safety Class: B | File: scripts/main.py | Traces to: FR-01, FR-10

Endpoints:

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | /api/health | Returns system status |
| POST | /api/analyze | Accepts image upload, returns complete 6-level JSON analysis |

Request: multipart/form-data, PNG or JPG image Response: HTTP 200, application/json Error: HTTP 400 for invalid image; HTTP 500 for inference failure

Startup: All three YOLOv8 models loaded into memory at startup. Not reloaded per request.

### 6.10 React Frontend

Safety Class: B | File: frontend/src/App.js | Traces to: FR-10

Design:

- File input accepts PNG and JPG (accept="image/*")

- Sends image to POST /api/analyze via Axios

- Displays all 6 level results on response

- Renders annotated X-ray decoded from base64 response field

- Displays loading state and error message on failure

## 7. Known Design Limitations

Non-Chest X-Ray Input The system was trained exclusively on standard chest X-rays. Uploading other image types — leg X-rays, MRI scans, CT scans, ultrasound images, or non-medical images — will produce unreliable results. The models have no awareness of input type and will attempt to analyze any image uploaded. Input validation to reject non-chest X-rays is listed as a future enhancement.

Level 3 Detection Below Target Detection achieved 53.1% mAP50 against a 70% target. Opacity boundaries on chest X-rays are inherently ambiguous — even experienced radiologists sometimes disagree on exact boundaries. With only 6,012 training images (detection tasks typically require 50,000+), this performance is expected even with a medium-sized model (YOLOv8m, 25.8M parameters). Five retraining iterations were conducted across varying model sizes, image resolutions, and augmentation strategies. The performance ceiling is determined by dataset size rather than training configuration.

OpenCV-Based Lung Segmentation Level 4 uses an OpenCV multi-threshold approach rather than a deep learning segmentation model. It works reliably across standard PA X-rays but may produce less precise lung boundaries on images with unusual positioning, high contrast variation, or visible medical equipment. The fallback to center 80% ensures Level 4 always receives a valid input.

Standard PA X-Rays Only All three AI models were trained exclusively on standard Posterior-Anterior (PA) chest X-rays. Portable bedside X-rays, AP-view X-rays, and ICU X-rays may produce lower confidence scores.

Dataset Scope Training data was sourced from public Kaggle datasets. These may not represent the full diversity of real-world clinical imaging across different equipment, patient populations, and geographic regions.

## 8. Traceability

| Functional Requirement | Module | Section |
| --- | --- | --- |
| FR-01 — Image Input | FastAPI Backend + React Frontend | 6.9, 6.10 |
| FR-02 — Binary Classification | Level 1 | 6.3 |
| FR-03 — Disease Classification | Level 2 | 6.4 |
| FR-04 — Disease Probability Scores | Level 2 | 6.4 |
| FR-05 — Opacity Detection | Level 3 | 6.5 |
| FR-06 — Lung Laterality Determination | Level 3 | 6.5 |
| FR-07 — Affected Area Calculation | Level 4 + Lung Segmentation | 6.2, 6.6 |
| FR-08 — Severity Classification | Level 5 | 6.7 |
| FR-09 — Clinical Recommendation | Level 6 | 6.8 |
| FR-10 — Analysis Output | FastAPI Backend | 6.9 |